WiFiDeck

User Guide  ·  functions, monitor mode, testing & resources

A complete tour of WiFiDeck: what every function does, how to test it, what to expect, a deep take on monitor mode, and curated learning resources so you can go deeper on the Wi-Fi security concepts underneath.

The WiFiDeck dashboard — live adapter state, scan, capture, crack, defense.

LEGAL FIRST.  Everything offensive here (monitor capture, deauth, handshake/PMKID cracking) is legal ONLY on networks you own or have written permission to test. In Canada see Criminal Code s. 342.1 (unauthorized use of a computer) and s. 430(1.1) (mischief to computer data). Use your own lab AP.

# 1. What WiFiDeck is

A local, browser-based command center for a USB Wi-Fi adapter (the ALFA AWUS036ACH / RTL8812AU, and similar). It shows live adapter state, scans for networks, captures WPA handshakes / PMKIDs, verifies and cracks them against a wordlist, and runs authorization-gated active tests — all from one dark, real-time dashboard that talks to a small FastAPI backend over WebSockets.

It runs on two platforms from one codebase — it detects the OS and adapts:

|  | macOS (native, primary) | Linux (VM or bare metal) |
| --- | --- | --- |
| RF backend | libusb driver (rtl8812au-macos) | kernel driver + nl80211 / iw / airodump |
| Managed mode (join networks) | No (not possible) | Yes |
| Monitor RX (sniff frames) | Yes (2.4 + 5 GHz) | Yes (with a working driver) |
| Raw injection (deauth) | Yes (primitive proven) | Yes |
| "Scan" | brief monitor capture -> beacons | managed nmcli OR monitor airodump |
| Root needed? | No (runs as your user) | Yes (mode/capture/deauth/share) |

macOS forbids third-party Wi-Fi kexts on Apple Silicon, so a userspace libusb driver can't be a system Wi-Fi interface — hence no managed mode. On Linux, monitor RX needs a driver that truly delivers frames: the in-tree rtw88 advertises monitor for the RTL8812AU but delivers no RX — use the aircrack rtl8812au DKMS driver on a kernel <= 6.15.

# 2. The core concept: MANAGED vs MONITOR mode

This is the single most important idea in the whole tool.

### Managed mode (station / client)

The normal way a Wi-Fi card works. The card associates with one access point, follows its channel, and the OS handles encryption, so you get a normal internet connection. You can join/leave networks, scan by asking the OS (nmcli on Linux), and see your link's SSID / signal / IP / TX-rate. What you CANNOT do: see other people's raw 802.11 frames — the card only hands the OS the decrypted traffic for the network it joined, so handshakes and management frames are filtered out.

### Monitor mode (RFMON / promiscuous 802.11)

The card stops associating and reports every raw 802.11 frame it hears on the channel it's parked on — beacons, probes, association and authentication/EAPOL (handshake) frames, data frames, from ALL nearby APs and clients, each wrapped in a radiotap header (channel, signal, rates). This is the mode that makes sniffing and capture possible. Trade-offs: no internet while in monitor; you only hear one channel at a time (a capture on ch 6 will not see an AP on ch 44 — set the card to the target's channel); and on this adapter monitor can't enumerate SSIDs by name the way managed does.

### How WiFiDeck bridges the gap

Linux: the instant you switch MANAGED -> MONITOR, the backend first runs one fresh managed scan and snapshots it. That remembered list (each entry keeps its channel) fills the deauth/capture pickers while you're in monitor. A scan failure never blocks the switch.

macOS: no managed mode to snapshot, so the Scan (monitor) panel does a brief (~5 s) capture on a channel, parses the beacons it heard, and fills the Target picker.

Practical model:  managed = "what network am I on?";  monitor = "what frames can I hear on this one channel?".  Capture always happens in monitor, on the target's channel.

### How to test the mode switch (Linux, real hardware)

1. Run  WIFIDECK_SUDO=1 ./wifideck  and open the dashboard.

2. Adapter status shows MANAGED. Scan -> you see the network list.

3. Click MONITOR in Mode Control. Expect a brief transition, then status shows MONITOR, the top-rail mode pill turns amber, and the Target picker is populated from the snapshot taken a second earlier.

4. Click MANAGED to switch back; the network list refreshes live again.

A second switch requested mid-transition returns HTTP 409 (busy) by design — the state machine serializes switches so the radio can't be corrupted. On macOS there is no MANAGED/MONITOR toggle (the UI hides it because the radio reports managed:false); use the Scan (monitor) panel instead.

# 3. Every function — how to test, what to expect

Fastest way to explore ALL of them with no hardware is mock mode:

WIFIDECK_MOCK=1 ./wifideck      # MOCK DATA badge; every panel has fixtures

The capture pipeline, end to end.

### 3.1 Adapter Status

Endpoint: GET /api/status · WS /ws/status

What: Live USB presence, driver, interface, mode, link state, SSID, IP, signal (dBm), TX-rate, and a health verdict (OK / DISCONNECTED / DEGRADED, incl. the -71 dBm weak-signal flag).

How to test: Just open the app; unplug the adapter (Linux).

What to expect: The card updates the instant anything changes (streamed, not polled); the banner flips to ADAPTER DISCONNECTED within a beat.

### 3.2 Mode Control

Endpoint: POST /api/mode

What: The MANAGED <-> MONITOR switch (Linux). Hidden on monitor-only radios (macOS).

How to test: See section 2.

What to expect: See section 2.

### 3.3 Scan / Network Table

Endpoint: WS /ws/scan · POST /api/scan/once

What: The list of networks — managed nmcli, monitor airodump (Linux), or the macOS beacon-scan. Columns: SSID, signal, CH, band, security, BSSID; sort, filter, filter by band.

How to test: Scan near any AP.

What to expect: For an enterprise SSID you'll see the same name on several rows — normal (multiple APs + multi-BSSID virtual APs); each row is a distinct BSSID+channel you can target. The security column classifies WPA2 / WPA3 / WPA3-transition / OWE / OPEN / 802.1X.

### 3.4 Posture (security classification)

What: Flags what each network's security means for capture — WPA2-PSK is offline-crackable if you catch a handshake; WPA3-SAE is NOT; WPA3-transition can often be downgraded to WPA2; OWE/OPEN have no PSK to crack.

How to test: Look at the SECURITY chips.

What to expect: Color-coded tone + a one-line 'what this means' note on the selected target.

### 3.5 Target selector

What: Pick the network once; the choice (BSSID + channel) is shared by deauth and guided capture, so you don't re-select per function.

How to test: Pick a row.

What to expect: Confirmation chips (SSID · ch · band · security · BSSID). If empty in monitor: 'switch to MANAGED once to scan, then come back.'

### 3.6 Connect

Endpoint: POST /api/connect

What: (Linux, managed) click an SSID to join/leave; NetworkManager stores the password.

How to test: Connect to your own AP.

What to expect: Status shows the SSID + an IP; a Disconnect button on the active row.

### 3.7 Capture

Endpoint: POST /api/capture · WS /ws/capture

What: The heart of it. Handshake mode runs airodump-ng locked to the target channel and waits for a client's 4-way handshake (EAPOL M1-M4) — pair with a deauth to force a reconnect. PMKID mode runs hcxdumptool to solicit a PMKID straight from the AP — no client needed.

How to test: Switch to MONITOR (Linux) -> set the target's channel -> start a handshake capture -> briefly deauth a phone on that AP -> watch frames climb. Or start PMKID and wait.

What to expect: A live session with a frame counter and, on success, HANDSHAKE ✓ or PMKID ✓ and an exportable .pcap. Mock mode fabricates a session so you can see the whole flow.

### 3.8 Import pcap

Endpoint: POST /api/capture/import

What: Adopt a pcap captured elsewhere (e.g. the macOS capture.py -o out.pcap) as a capture session — copied in, scanned for handshake/PMKID, recorded to history — so it flows through verify -> crack -> report. This is the macOS capture bridge.

How to test: Capture a pcap on macOS, paste its path (+ optional BSSID), Import.

What to expect: 'imported — handshake: yes/no', and a new History row.

### 3.9 Verify

What: tshark confirms a capture actually contains a complete 4-way handshake (M1-M4) or a PMKID BEFORE you waste time cracking.

How to test: Import/capture, then verify.

What to expect: An explicit M1-M4 / PMKID readout; a partial handshake is flagged as not crackable.

### 3.10 Cracking

Endpoint: POST /api/crack · WS /ws/crack

What: Run aircrack-ng or hashcat (mode 22000, the modern WPA format) against a wordlist. Scope-gated, live progress.

How to test: Point it at a wordlist (e.g. a tiny list containing your lab AP's known password) and your captured handshake.

What to expect: Live progress and, on success, the recovered key; the outcome is written to History.

Note: Cracking only recovers weak/guessable passphrases from a wordlist — it is NOT a magic WPA2 break, and it does NOT work on WPA3-SAE.

### 3.11 Active — Deauth

Endpoint: POST /api/active/deauth

What: Send 802.11 deauthentication frames to knock a client off so it reconnects and you catch its handshake. OFF by default; arm with WIFIDECK_SUDO=1 WIFIDECK_ENABLE_ACTIVE=1 ./wifideck.

How to test: Pick your target, confirm once.

What to expect: Every action is audited (who/when/what BSSID) and requires an explicit confirm.

Note: Deauth is a WPA2 technique; 802.11w / PMF (mandatory in WPA3) protects management frames, so deauth is rejected — which is exactly why WPA3 matters.

### 3.12 Guided flow

Endpoint: POST /api/flow

What: One gated workflow chaining it end to end: monitor -> capture -> (deauth) -> handshake -> export, with a step tracker.

How to test: Pick a target, start the flow.

What to expect: Each step lights up as it completes; the flow stops on the first failure and says why.

### 3.13 Stations

Endpoint: GET /api/stations

What: Clients (stations) seen associated to APs during a monitor scan — tells you which AP actually has clients worth deauthing.

How to test: Monitor-scan near an active AP.

What to expect: A client list with the BSSID each is talking to.

### 3.14 Watchdog

Endpoint: WS /ws/watchdog

What: Auto-recovers the ALFA's -71/USB-drop wedges (driver reload -> USB reset -> reconnect).

How to test: Enable it, then induce a drop.

What to expect: An events timeline showing the recovery steps.

### 3.15 WIDS-lite (defense)

Endpoint: WS /ws/wids

What: The defensive side — detects evil-twin APs (same SSID, new BSSID) and deauth floods, with an alerts timeline. Set a baseline of 'known good' first.

How to test: Baseline, then run a deauth from another device.

What to expect: A deauth-flood alert. Great for learning what an attack looks like from the defender's chair.

### 3.16 Anomaly · Metrics · Share · History · Report · Schedule · Integrations

Anomaly — flags odd signal/behaviour patterns.

Metrics — signal & TX-rate sparklines over time.

Share (Linux) — NAT the adapter's uplink to your host, with copyable route/DNS commands.

History (SQLite) — past capture sessions + crack outcomes, persisted across restarts. Test: import a pcap, restart the backend, reopen — the row is still there.

Report — an HTML summary of your sessions/findings.

Schedule — run captures/scans on a timer.

Integrations — webhook / ntfy / Slack notifications; the panel has a Send test button.

# 4. Testing playbooks

### 4.1 Zero-hardware tour (any OS, 2 minutes)

WIFIDECK_MOCK=1 ./wifideck

Click through every panel. A MOCK DATA badge is shown; nothing touches real radios. Best way to learn the UI and the capture->verify->crack->history flow.

### 4.2 Live recon on macOS (your own AP)

Set up the libusb driver once (see MACOS-SETUP).

./wifideck -> top rail shows macOS; Radio Doctor shows the libusb backend.

Scan (monitor) on your AP's channel -> APs fill the Target picker.

Capture with capture.py -o out.pcap on your channel -> Import pcap -> Verify -> Crack (against a wordlist you control).

### 4.3 Live recon on Linux (your own AP)

Working DKMS driver on a kernel <= 6.15 (see HARDWARE).

WIFIDECK_SUDO=1 WIFIDECK_ENABLE_ACTIVE=1 ./wifideck.

Scan (managed) -> pick your AP as Target -> switch MONITOR (channel remembered) -> Capture (handshake) -> Deauth your own phone once -> handshake ✓ -> Verify -> Crack.

### 4.4 What success and failure look like

Good capture: frame counter climbs; HANDSHAKE ✓ / PMKID ✓; pcap exportable; History row.

No handshake: counter climbs but no ✓ — usually wrong channel, no client on that AP, or PMF blocking deauth. Fix the channel first.

Crack 'not found': the passphrase isn't in your wordlist (expected for strong passwords), or the network is WPA3-SAE (not offline-crackable).

# 5. The honest truth: WPA2 vs WPA3

WPA2-PSK: capture a 4-way handshake (or a PMKID) -> run a dictionary/brute-force attack OFFLINE. You're only limited by how guessable the password is and your GPU. This is what WiFiDeck's capture->crack pipeline does.

WPA3-SAE ("Dragonfly"): replaces the crackable handshake with a zero-knowledge exchange — there is no captured value you can take offline and brute-force. No known practical offline crack of a correctly configured WPA3-SAE network. The real-world attacks are:

Transition-mode downgrade: many 'WPA3' networks also accept WPA2 for old clients; you attack the WPA2 side. Pure WPA3-only closes this.

Dragonblood (2019): side-channel/downgrade bugs in early WPA3 implementations — largely patched.

PMF/802.11w: WPA3 mandates protected management frames, so deauth doesn't work, which breaks the classic handshake-forcing trick.

Takeaway: WPA3 didn't 'get cracked' — it removed the offline-crackable artifact. The lesson is defensive as much as offensive.

# 6. Additional resources

### Books

Kali Linux Wireless Penetration Testing (Buchanan / Ramachandran, Packt)  —  hands-on airmon/airodump/aircrack — closest to what WiFiDeck automates.

The Hacker Playbook 3 (Peter Kim)  —  practical methodology incl. a Wi-Fi chapter.

802.11 Wireless Networks: The Definitive Guide (Matthew Gast, O'Reilly)  —  the protocol bible: frames, management vs data, the 4-way handshake — read this to truly understand monitor mode.

Hacking Exposed Wireless (Cache / Wright / Liu)  —  attacks + defenses across Wi-Fi / Bluetooth.

CWNA / CWSP Study Guides (Sybex)  —  vendor-neutral certs; CWNA for RF/802.11 fundamentals, CWSP for security depth.

### Videos / channels

Vivek Ramachandran — WLAN Security Megaprimer  —  https://www.youtube.com/playlist?list=PLA25FCB53DD770262  —  the classic free monitor-mode/aircrack video course.

David Bombal — Wi-Fi hacking  —  https://www.youtube.com/@davidbombal  —  ALFA adapters, hashcat, tool-author interviews; beginner-friendly.

NetworkChuck  —  https://www.youtube.com/@NetworkChuck  —  approachable Wi-Fi / Kali intros.

Hak5  —  https://www.youtube.com/@hak5  —  Wi-Fi Pineapple / deauth demos; good for seeing attacks live.

DEF CON talks  —  https://www.youtube.com/@DEFCONConference  —  search 'WPA3', 'Dragonblood', '802.11'.

### Websites / docs / tools

Aircrack-ng wiki  —  https://www.aircrack-ng.org/doku.php  —  canonical docs for the tools under the hood.

hashcat wiki  —  https://hashcat.net/wiki/  —  the WPA/PMKID mode 22000 format and usage.

hcxdumptool  —  https://github.com/ZerBea/hcxdumptool  —  clientless PMKID capture (WiFiDeck's PMKID mode).

Wireshark  —  https://www.wireshark.org/  —  open a WiFiDeck .pcap, filter 'eapol' to see M1-M4 yourself.

Wi-Fi Alliance — security  —  https://www.wi-fi.org/discover-wi-fi/security  —  official WPA3 overview.

Dragonblood (Mathy Vanhoef)  —  https://wpa3.mathyvanhoef.com/  —  WPA3 attack research; also read up on KRACK (his WPA2 attack).

Kali Linux docs  —  https://www.kali.org/docs/  —  drivers, wireless setup.

aircrack-ng/rtl8812au (DKMS driver)  —  https://github.com/aircrack-ng/rtl8812au  —  the Linux driver for this chip.

### Safe places to practice (don't touch networks you don't own)

Your own lab AP — a cheap travel router or spare home router with a known weak passphrase you set is the ideal target.

Hack The Box / TryHackMe — guided labs (mostly host/web, some wireless theory).

CTFs — search 'wireless CTF', '802.11 pcap challenge'; practice handshake analysis in Wireshark without transmitting anything.

### Concepts worth searching as you go

802.11 frame types · 4-way handshake / EAPOL · PMKID · radiotap header · beacon vs probe · BSSID vs SSID vs ESSID · multi-BSSID / virtual AP · monitor mode / RFMON · channel vs frequency / 2.4 vs 5 GHz · PMF / 802.11w · WPA2-PSK vs WPA3-SAE · hashcat mode 22000 · deauthentication attack · evil twin · KRACK · Dragonblood.

# 7. Troubleshooting quick hits

No networks in the Target picker (monitor).  Switch to MANAGED once to snapshot a scan (Linux), or use Scan (monitor) (macOS). Confirm you're on the target's channel.

Handshake never completes.  Wrong channel, no client on that AP, or PMF is blocking deauth (WPA3). Try PMKID.

'iw: No such file / managed failed' on macOS.  Expected — macOS has no managed mode; the toggle is hidden and Scan is monitor-based.

Adapter 'off the bus' after a capture (macOS).  A known post-capture wedge — physically replug the ALFA.

Crack finds nothing.  Password not in the wordlist, or it's WPA3-SAE.

WiFiDeck · localhost console · generated from docs/USER-GUIDE.md